# Saikei Civil — Site Grading & Earthwork Implementation Spec

Status: Draft — Agent implementation guide Target repo path: C:\GitHub\ifcopenshell\src\bonsai\bonsai\ (branch: saikei-dev) Scope: Non-linear site grading (pads, parking lots, ponds, infield areas) Companion doc: Saikei_Grading_Earthwork_Research.md (commercial tool survey — background reading)

## 1. Purpose and Model

Saikei Civil models site grading using the AutoCAD Civil 3D pattern, implemented with IFC 4.3 entities as the native persistence layer. Civil 3D’s model is the most widespread in US practice and the most legible to practicing civil engineers; adopting it means minimal conceptual retraining for the target user.

This document is the implementation spec for agents. It defines the data model, the 3-layer architecture mapping, the IFC entities, the operator set, and the sprint breakdown. For background on why Civil 3D’s model was chosen over alternatives, see the companion research doc.

### Core Civil 3D concepts Saikei adopts

| Civil 3D concept | Role | Saikei class |
| --- | --- | --- |
| Surface (TIN) | Existing or proposed ground | CivilSurface |
| Breakline | Constrained edge in a surface | Breakline |
| Feature Line | 3D polyline with elevations — grading footprint | FeatureLine |
| Grading Criteria | Rule for projecting a slope from a feature line | GradingCriteria |
| Grading | Feature line + criteria + target → slope projection geometry | GradingObject |
| Grading Group | Collection of gradings composing one feature; produces one surface + volumes | GradingGroup |
| Cut/Fill Report | Earthwork quantities between two surfaces | VolumeResult |

Saikei does not implement the Civil 3D “site” concept for automatic topological interaction between grading objects. That behavior is fragile in Civil 3D itself, and its absence can be worked around by disciplined feature-line authoring. Grading groups are standalone.

### The Civil 3D grading workflow Saikei supports

The MVP target is this workflow, identical to Civil 3D’s:

- Load or create existing ground surface (from XYZ points, CSV, or IFC import).

- Draw or import a feature line representing the grading footprint.

- Assign elevations to feature line vertices — either manually, from the surface (drape), or via grade/elevation editing.

- Create a grading group targeting the existing surface.

- Author or select a grading criteria (e.g., “slope to surface, 3:1 fill / 2:1 cut”).

- Apply the criteria to the feature line — Saikei computes slope projection, daylight line, and the proposed surface for the grading object.

- Add more gradings to the group as needed; the group surface composes.

- Compute earthwork volumes between existing and proposed surface.

- Export to IFC.

## 2. Mapping to IFC 4.3

Each Civil 3D concept maps to a standard IFC 4.3 entity. No schema extensions are required.

### 2.1 Surfaces — IfcTriangulatedIrregularNetwork

IfcTriangulatedIrregularNetwork (subtype of IfcTriangulatedFaceSet) is the canonical IFC 4.3 terrain representation. Key facts:

- Closed must be FALSE — it’s an open surface, not a solid.

- One unique Z per (X, Y) — draped surfaces only. Fine for typical civil site grading; overhanging/vertical faces (retaining walls, steep cut faces) are a known limitation handled via separate wall entities (see §10, future considerations).

- Triangles counterclockwise from above (right-hand rule).

- Flags list: one integer per triangle encoding breakline membership or hole/void status.

Saikei Flags encoding convention:

- 0 = normal interior triangle

- positive N = triangle has at least one edge on breakline with Saikei breakline ID N

- -1 = Hole (excluded, may fall back on another surface)

- -2 = Void (excluded, no fallback)

Known limitation: the IFC Flags list is per-triangle, not per-edge. A triangle can straddle a breakline with only one of its three edges actually constrained; the encoding cannot distinguish which edge. If a triangle has edges from multiple breaklines, only one breakline ID can be stored per the schema. Round-trip preserves per-triangle breakline membership, not per-edge identity. Full breakline geometry is persisted separately as IfcAnnotation polylines (see §2.3) to preserve editability on re-open.

### 2.2 Host entities — existing vs. proposed surfaces

Saikei distinguishes existing from proposed ground via the host entity type, not via a property on a shared host:

| Surface kind | Host IFC entity | Notes |
| --- | --- | --- |
| Existing ground | IfcGeotechnicalStratum | TIN attached as shape representation. Pset_SoilMechanicalLayer for soil properties. |
| Proposed ground (per grading group) | IfcEarthworksFill with PredefinedType=SUBGRADE | TIN attached as shape representation. The fill entity also carries the solid fill volume representation (see §2.4). |
| Composite site proposed (all groups merged) | IfcEarthworksFill with PredefinedType=SUBGRADE | Top-level site-proposed surface; aggregates contained group fills via IfcRelAggregates. |

This resolves the previous ambiguity where proposed surfaces were loosely assigned to IfcGeographicElement. IfcGeographicElement is semantically for natural geographic features and is the wrong host for a designed surface.

Important: surface ≠ fill solid. A proposed IfcEarthworksFill carries two distinct geometric representations: 1. The proposed surface (top face of the design) — IfcTriangulatedIrregularNetwork in a SurfaceModel ShapeRepresentation. 2. The fill volume solid (the dirt between proposed and existing) — IfcPolygonalFaceSet with Closed=TRUE in a Brep ShapeRepresentation.

Both are attached to the same IfcEarthworksFill entity in different IfcShapeRepresentation blocks. IfcProductDefinitionShape.Representations holds the list.

### 2.3 Feature lines and grading criteria — IFC persistence

These are Civil 3D-native concepts with no directly-matching IFC 4.3 entity. Saikei commits to specific persistence strategies:

Feature lines are persisted as IfcAlignment entities with a simplified representation: - IfcAlignment carries a 3D IfcPolyline or IfcIndexedPolyCurve as its shape representation (IFC 4.3 explicitly lists IfcPolyline as a valid IfcAlignment representation for 3D polyline alignments from survey data). - No IfcAlignmentHorizontal / IfcAlignmentVertical subdivision is required for feature lines — the polyline representation suffices. - Pset_SaikeiFeatureLineCommon carries Saikei-specific metadata (closed/open, source origin, grading group membership). - This reuses the existing alignment IFC infrastructure rather than inventing new entities.

Breakline source geometry is persisted as IfcAnnotation polylines separate from the derived TIN. This preserves editability — on file re-open, the breakline polylines are still present and retriangulation can be re-run. Without this, the TIN would load but the breaklines that produced it would be lost.

Grading criteria are persisted as reusable IfcPropertySetTemplate definitions in the project, instantiated as IfcPropertySet on each grading group that uses them. This matches Civil 3D’s “grading criteria set” model where criteria are named and reusable across multiple groups. The template carries the parameter structure (target_kind, cut_slope, fill_slope, max_distance, etc.); instances bind to specific values.

### 2.4 Cut and fill volumes — IfcEarthworksCut / IfcEarthworksFill

IfcEarthworksCut is a subtype of IfcFeatureElementSubtraction. Semantically it represents the act of excavation; geometrically it is the void created. It relates to the host stratum via IfcRelVoidsElement (the same relationship doors use to void walls).

Shape representation for cut volumes: IfcPolygonalFaceSet with Closed=TRUE in a ShapeRepresentation with RepresentationType='Brep'. This is the IFC 4.3 preferred lightweight closed solid. Pattern matches IfcOpeningElement: the cut has its own renderable solid geometry and establishes a void relationship via IfcRelVoidsElement. These are complementary, not alternative.

IfcEarthworksFill is a subtype of IfcEarthworksElement, which is a subtype of IfcBuiltElement → IfcElement. (Not IfcElementAssembly — that is a sibling, not an ancestor.) PredefinedType enum covers EMBANKMENT, SUBGRADE, SUBGRADEBED, SLOPEFILL, BACKFILL, COUNTERWEIGHT, TRANSITIONSECTION.

Shape representation for fill volumes: also IfcPolygonalFaceSet with Closed=TRUE. Same Brep representation type.

Important: IfcEarthworksCut does not represent the excavated material. For a cut + fill scenario where excavated dirt is reused as fill, Saikei authors separate IfcEarthworksCut and IfcEarthworksFill entities with no direct IFC link between them (material tracking is a resource concept not in IFC 4.3).

### 2.5 Slope fill and interior fill as distinct entities

Within a grading group, the composite proposed surface is composed of multiple distinct regions that have different material treatment and quantities. Saikei authors each as a separate IfcEarthworksFill:

- Slope fill — the sloped tie from the feature line out to the daylight line. PredefinedType=SLOPEFILL. One per grading object.

- Interior fill — the floor of the pad, pond, or infield area enclosed by feature lines. PredefinedType=SUBGRADE. One per grading group (if interior_fill != "none").

- Group composite — the IfcEarthworksFill that aggregates the per-object slope fills and interior fill via IfcRelAggregates. PredefinedType=SUBGRADE.

This is takeoff-friendly: contractors can quantify slope fill (typically borrow/waste) separately from subgrade fill (often imported structural fill or native compacted).

### 2.6 Grading groups — IfcGroup with ObjectType="GradingGroup"

A grading group is a logical collection of cuts and fills that together compose one designed feature. The IFC entity is IfcGroup with ObjectType="GradingGroup", with members attached via IfcRelAssignsToGroup.

Not IfcBuiltSystem. The IRROADWP3 Annex I example diagrams suggest IfcBuiltSystem[EARTHWORK], but IfcBuiltSystemTypeEnum does not include EARTHWORK as a valid value (the enum is EROSIONPREVENTION, FENESTRATION, FOUNDATION, LOADBEARING, MOORING, OUTERSHELL, PRESTRESSING, RAILWAYLINE, RAILWAYTRACK, REINFORCING, SHADING, TRACKCIRCUIT, TRANSPORT, USERDEFINED, NOTDEFINED). Using IfcBuiltSystem[USERDEFINED] with ObjectType="Earthwork" works schema-wise but is a semantic stretch — IfcBuiltSystem is defined as “built elements forming a particular structural or service part of a facility,” which site grading is not.

IfcGroup is the parent type of IfcSystem (and therefore IfcBuiltSystem). It has no PredefinedType constraint, accepts arbitrary ObjectType strings, and will round-trip through any compliant viewer without semantic confusion. This is the simpler, honest mapping.

### 2.7 Spatial hierarchy

IfcProject
└── IfcSite  [IfcRelAggregates from IfcProject]
    │
    ├── IfcGeomodel  [IfcRelContainedInSpatialStructure from IfcSite]
    │   │
    │   │   NOTE: IfcGeomodel is an IfcElement (via IfcGeotechnicalAssembly →
    │   │         IfcGeotechnicalElement → IfcElement), NOT an IfcSpatialElement.
    │   │         Therefore it uses IfcRelContainedInSpatialStructure, not
    │   │         IfcRelAggregates. This is consistent with how IfcAlignment
    │   │         (also an IfcElement, via IfcLinearPositioningElement) sits
    │   │         under IfcSite.
    │   │
    │   ├── IfcGeotechnicalStratum  [IfcRelAggregates from IfcGeomodel]
    │   │   └── Rep: IfcTriangulatedIrregularNetwork (existing ground TIN)
    │   │
    │   ├── IfcEarthworksCut [EXCAVATION]  [IfcRelAggregates from IfcGeomodel]
    │   │   ├── Rep: IfcPolygonalFaceSet Closed (cut volume solid)
    │   │   └── Voids IfcGeotechnicalStratum via IfcRelVoidsElement
    │   │
    │   └── IfcEarthworksFill [SUBGRADE] (composite site proposed)
    │       [IfcRelAggregates from IfcGeomodel]
    │       ├── Rep 1: IfcTriangulatedIrregularNetwork (proposed TIN)
    │       ├── Rep 2: IfcPolygonalFaceSet Closed (fill volume solid)
    │       └── Aggregates per-group fills via IfcRelAggregates
    │
    └── IfcGroup ObjectType="GradingGroup"  [IfcRelContainedInSpatialStructure
        │                                     from IfcSite]
        ├── Contains IfcEarthworksCut instances via IfcRelAssignsToGroup
        ├── Contains IfcEarthworksFill instances (slope + interior) via IfcRelAssignsToGroup
        └── Contains IfcAlignment feature lines via IfcRelAssignsToGroup

Relationship summary:

- IfcProject → IfcSite: IfcRelAggregates (spatial structure)

- IfcSite → IfcGeomodel: IfcRelContainedInSpatialStructure (Geomodel is an Element)

- IfcSite → IfcGroup (grading group): IfcRelContainedInSpatialStructure

- IfcGeomodel → IfcGeotechnicalStratum: IfcRelAggregates (element decomposition)

- IfcGeomodel → IfcEarthworksCut: IfcRelAggregates (element decomposition)

- IfcGeomodel → IfcEarthworksFill: IfcRelAggregates (element decomposition)

- IfcEarthworksFill → sub-IfcEarthworksFill (group composite → slope + interior): IfcRelAggregates

- IfcGeotechnicalStratum ← IfcEarthworksCut: IfcRelVoidsElement (void relation)

- IfcGroup → members: IfcRelAssignsToGroup

### 2.8 Object placement

All earthwork products (IfcGeotechnicalStratum, IfcEarthworksCut, IfcEarthworksFill, IfcGeomodel) use: - IfcLocalPlacement with identity IfcAxis2Placement3D (origin (0,0,0), axes aligned to project coordinate system). - Absolute world positioning is handled by IfcMapConversion at the project level — the same pattern the existing alignment tool uses.

This is prescriptive. Implementors should not use non-identity local placements for earthwork entities; it breaks the georeferencing contract shared with alignments.

### 2.9 Bounding box representation

Every terrain surface authored by Saikei attaches an IfcBoundingBox representation alongside the SurfaceModel representation: - RepresentationIdentifier='Box', RepresentationType='BoundingBox'. - Provides IfcBoundingBox.Corner (min x,y,z) + XDim, YDim, ZDim.

This enables viewer-side LOD and culling without forcing the full TIN parse for every frame. Site-scale surfaces (multi-hectare) are large enough that this matters in practice.

### 2.10 Georeferencing

Horizontal: IfcMapConversion with SourceCRS=IfcGeometricRepresentationContext and TargetCRS=IfcProjectedCRS — the established Saikei pattern.

Vertical: IfcProjectedCRS.VerticalDatum is required (not optional) for surfaces. Values: NAVD88, NGVD29, WGS84_ELLIPSOIDAL, or project-specific string. IfcMapConversion.OrthogonalHeight encodes the geoid-ellipsoid separation at the site centroid. Without these, 0.1 ft vertical errors accumulate and drainage design becomes unreliable through IFC round-trip.

This is a shared gap with the current alignment implementation; fixing it here for surfaces creates the pattern for alignments to adopt.

## 3. Property Sets and Quantity Sets

### 3.1 Standard IFC 4.3 Psets — use these first

| Entity | Standard Pset | Use for |
| --- | --- | --- |
| IfcGeotechnicalStratum | Pset_GeotechnicalStratumTypeCommon | StratumType, StratumFunction, IsProposed |
| IfcGeotechnicalStratum | Pset_SoilMechanicalLayer | Bearing capacity, plasticity index, moisture content |
| IfcEarthworksFill | Pset_EarthworksFillCommon | ContaminationLevel, FillMaterial, IsTopsoilStripped |
| IfcEarthworksCut | Pset_EarthworksCutCommon | CuttingType, ContaminationLevel, IsTopsoilStripped |

Populate these on every relevant entity. Do not duplicate their properties in custom Psets.

### 3.2 Standard IFC 4.3 Qtos — volumes live here

| Entity | Qto | Quantities |
| --- | --- | --- |
| IfcEarthworksFill | Qto_EarthworksFillBaseQuantities | Length, Width, Depth, CompactedVolume, LooseVolume |
| IfcEarthworksCut | Qto_EarthworksCutBaseQuantities | Length, Width, Depth, UndisturbedVolume, LooseVolume, Weight |

All earthwork volumes Saikei computes go here, not in custom Psets. QTO tools (Solibri, CostX, iTWO) query these standard Qtos and will find Saikei volumes.

### 3.3 Saikei custom Psets — only for what standards don’t cover

Naming follows bSI convention: Pset_<Organization><Topic> with no internal underscores in the topic portion.

| Pset | Attached to | Purpose |
| --- | --- | --- |
| Pset_SaikeiGradingSource | IfcGroup (grading group) | Source metadata: criteria reference, author, timestamp, grading group version |
| Pset_SaikeiGradingSurface | IfcTriangulatedIrregularNetwork host entity | Triangulation tolerance, breakline count, vertex count, boundary polygon reference |
| Pset_SaikeiGradingCriteria (template) | IfcPropertySetTemplate | Reusable criteria definitions: target_kind, cut_slope, fill_slope, max_distance, retaining_wall_at_limit |
| Pset_SaikeiGradingShrinkSwell | IfcEarthworksFill, IfcEarthworksCut | ShrinkFactor, SwellFactor (not in standard Qtos — only Compacted/Loose volumes are) |
| Pset_SaikeiFeatureLineCommon | IfcAlignment (feature line) | IsClosed, Source, GradingGroupGuid, ElevationSource |
| Pset_SaikeiGradingAlignment | IfcGroup (grading group), IfcEarthworksFill | AlignmentGuid, StartStation, EndStation — for grading adjacent to a corridor |

### 3.4 Classification

Every IfcEarthworksCut and IfcEarthworksFill authored by Saikei is classified via IfcRelAssociatesClassification using OmniClass Table 22 (Work Results). Default codes:

| Element | OmniClass Code | Title |
| --- | --- | --- |
| Bulk excavation cut | 22-07 31 00 | Earthwork |
| Structural excavation | 22-07 31 16 | Excavation and Fill |
| Embankment / general fill | 22-07 31 23 | Fill |
| Topsoil stripping | 22-07 31 14 | Site Clearing (topsoil removal) |

These map to CSI MasterFormat Division 31 codes that US civil engineers and estimators recognize. Saikei authors the classification automatically based on the PredefinedType of the earthwork entity. Users can override or add additional classifications (Uniclass, MasterFormat direct) via UI.

Classification matters for interop with QS/cost-estimating software that queries by classification code, not by IFC entity type.

## 4. Architecture

Saikei grading follows the mandatory Bonsai 3-layer architecture. Dion’s clarification is strictly enforced: Core = workflow orchestration only (no math, no bpy, no IFC creation); Tool = all implementations; UI = operators and panels.

### 4.1 Module layout

src/bonsai/bonsai/
├── core/
│   ├── alignment.py          (existing)
│   ├── surface.py            (new — workflow orchestration for TIN surfaces)
│   ├── grading.py            (new — orchestration for feature lines / criteria / gradings)
│   └── earthwork.py          (new — orchestration for volume calcs and IFC export)
├── tool/
│   ├── alignment.py          (existing)
│   ├── surface.py            (new — TIN math, breakline CDT, IFC authoring)
│   ├── grading.py            (new — slope projection, group composition)
│   └── earthwork.py          (new — prismoidal volumes, cut/fill map)
└── bim/module/
    ├── alignment/            (existing)
    ├── surface/              (new — surface panel, operators, props)
    ├── grading/              (new — grading panel, operators, props)
    └── earthwork/            (new — earthwork panel, operators, props)

### 4.2 Operator prefixes

Following the established CIVIL_OT_* / CIVIL_PT_* convention:

- Surface: CIVIL_OT_surface_*, CIVIL_PT_surface_*

- Grading: CIVIL_OT_grading_*, CIVIL_PT_grading_*

- Earthwork: CIVIL_OT_earthwork_*, CIVIL_PT_earthwork_*

All operators inherit tool.Ifc.Operator and implement _execute() for automatic undo/redo.

### 4.3 Core-to-Tool pattern

Core never invents API paths. The existing core/alignment.py pattern is: core orchestrates by calling tool methods; tool methods either call real ifcopenshell.api.* functions or author entities directly via ifc_file.create_entity(). Core does not call tool.Ifc.run("some.fabricated.path").

There is no ifcopenshell.api.surface, no ifcopenshell.api.grading, no ifcopenshell.api.earthwork. These would need to be built and upstreamed separately, which is out of scope for this sprint. Until they exist, tool methods author IFC entities directly via ifc_file.create_entity().

See §6 for the corrected core/tool code patterns.

### 4.4 Surface math backend

- scipy.spatial.Delaunay — unconstrained Delaunay triangulation (point cloud → TIN, no breaklines).

- shapely.constrained_delaunay_triangles (Shapely 2.1+) — constrained Delaunay, honors breaklines and hole/void polygons.

The backend is abstracted behind a Triangulator class in tool/surface.py so it can be swapped if performance needs dictate (e.g., if a site exceeds ~1M triangles, we’d consider a compiled library — not MVP concern).

### 4.5 Shared Bonsai tools reused

- tool.Ifc — get(), link(), unlink() for IFC entity access.

- tool.Loader.create_generic_shape() — IfcTriangulatedIrregularNetwork → Blender mesh via IfcOpenShell geometry engine.

- tool.Georeference — enh2xyz / xyz2enh for coordinate transforms. Surfaces store engineering coordinates in IFC, render near-origin in Blender for 32-bit float precision.

- tool.Collector.assign() — spatial hierarchy placement.

- tool.Blender — validate_shader_batch_data, scale_font_size for annotation.

## 5. Data Model

Data primitives live in tool/surface.py, tool/grading.py, tool/earthwork.py. They are not Core-facing — Core only sees them via tool-class methods.

# tool/surface.py

from dataclasses import dataclass, field
from typing import Literal
import numpy as np
import shapely


@dataclass
class Breakline:
    """A 3D polyline that must be honored as constrained edges in a TIN.
    Persisted to IFC as IfcAnnotation with IfcPolyline representation."""
    guid: str
    name: str
    polyline: list[tuple[float, float, float]]   # (x, y, z) per vertex
    kind: Literal["standard", "wall", "non_destructive", "proximity"]
    source: str                                    # "manual", "feature_line", "corridor_extract"
    ifc_annotation_id: int | None = None           # step id of IfcAnnotation


@dataclass
class CivilSurface:
    """An engineered or measured 3D surface — the central grading primitive."""
    guid: str
    name: str
    kind: Literal["existing", "proposed_group", "proposed_site"]
    points: np.ndarray                             # shape (N, 3), xyz coordinates
    triangles: np.ndarray                          # shape (M, 3), vertex indices into points
    triangle_flags: np.ndarray                     # shape (M,), IFC Flags list values
    breaklines: list[Breakline] = field(default_factory=list)
    outer_boundary: shapely.Polygon | None = None  # 2D, XY only
    holes: list[shapely.Polygon] = field(default_factory=list)
    voids: list[shapely.Polygon] = field(default_factory=list)

    # IFC linkage — depends on surface kind
    # existing:       host is IfcGeotechnicalStratum
    # proposed_group: host is IfcEarthworksFill [SUBGRADE] within a grading group
    # proposed_site:  host is IfcEarthworksFill [SUBGRADE] at the composite site level
    ifc_host_entity_id: int | None = None          # step id of host IfcProduct
    ifc_tin_representation_id: int | None = None   # step id of IfcTriangulatedIrregularNetwork
    ifc_bbox_representation_id: int | None = None  # step id of IfcBoundingBox

    metadata: dict = field(default_factory=dict)

# tool/grading.py

@dataclass
class FeatureLine:
    """A 3D polyline used as a grading footprint. Civil 3D's Feature Line analog.
    Persisted to IFC as IfcAlignment with IfcPolyline/IfcIndexedPolyCurve representation."""
    guid: str
    name: str
    vertices: list[tuple[float, float, float]]
    closed: bool                                   # most site pads are closed loops
    grading_group: str | None = None               # parent grading group guid
    ifc_alignment_id: int | None = None            # step id of IfcAlignment


@dataclass
class GradingCriteria:
    """A rule for projecting a slope from a feature line. Civil 3D's Grading Criteria analog.
    
    Civil 3D ships four base target kinds; Saikei MVP implements all four.
    Criteria are persisted as IfcPropertySetTemplate at project scope (reusable), with
    IfcPropertySet instances bound to the template on each grading group that uses the
    criteria.
    """
    guid: str
    name: str
    target_kind: Literal["surface", "elevation", "relative_elevation", "distance"]
    target_ref: str | float                        # surface guid, abs elevation, rel elev, or distance
    cut_slope: float                               # ratio (H:V), e.g., 2.0 means 2:1
    fill_slope: float                              # ratio (H:V), e.g., 3.0 means 3:1
    max_distance: float | None = None              # daylight cap; None = unlimited
    retaining_wall_at_limit: bool = False          # insert wall if cap hit before target reached
    ifc_template_id: int | None = None             # step id of IfcPropertySetTemplate


@dataclass
class GradingObject:
    """A feature line + criteria, producing slope projection geometry.
    
    Civil 3D's Grading analog. Has dynamic relationship to its footprint — edits to
    the feature line trigger rebuild. The slope projection produces a slope fill
    region persisted as IfcEarthworksFill [SLOPEFILL].
    """
    guid: str
    name: str
    footprint: FeatureLine
    criteria: GradingCriteria
    target_surface_guid: str | None                # for target_kind="surface"
    daylight_line: list[tuple[float, float, float]]  # computed — 3D polyline of tie points
    projection_triangles: np.ndarray               # computed — slope face triangles (M, 3)
    projection_points: np.ndarray                  # computed — xyz coords (N, 3)
    ifc_slope_fill_id: int | None = None           # step id of IfcEarthworksFill [SLOPEFILL]


@dataclass
class GradingGroup:
    """A collection of grading objects composing one graded feature.
    
    Civil 3D's Grading Group analog. Produces one composite proposed surface and
    one earthwork report. Persisted to IFC as IfcGroup with ObjectType="GradingGroup".
    """
    guid: str
    name: str
    members: list[GradingObject] = field(default_factory=list)
    interior_fill: Literal["none", "flat", "interpolate_from_boundary", "from_surface"] = "interpolate_from_boundary"
    interior_fill_source_guid: str | None = None   # for interior_fill="from_surface"
    output_surface_guid: str | None = None         # computed composite proposed surface
    target_surface_guid: str | None = None         # existing ground reference

    # IFC linkage
    ifc_group_id: int | None = None                # step id of IfcGroup
    ifc_composite_fill_id: int | None = None       # step id of aggregating IfcEarthworksFill
    ifc_interior_fill_id: int | None = None        # step id of interior IfcEarthworksFill [SUBGRADE]

Design note on interior_fill. Civil 3D’s grading groups have an ambiguous “infill” behavior that trips up new users. Saikei makes the interior fill strategy explicit — the engineer must pick one of four options when creating the group, and it’s visible as a property thereafter. The interior fill is authored as a distinct IfcEarthworksFill[SUBGRADE] entity, separate from the slope fills, enabling independent quantity takeoff (contractors typically specify different material and compaction for pad subgrade vs. slope fill).

# tool/earthwork.py

@dataclass
class VolumeResult:
    """Output of a TIN-to-TIN prismoidal volume calculation.
    
    Volumes are written to standard IFC Qtos:
      Qto_EarthworksCutBaseQuantities: UndisturbedVolume, LooseVolume
      Qto_EarthworksFillBaseQuantities: CompactedVolume, LooseVolume
    
    Shrink/swell factors are written to Pset_SaikeiGradingShrinkSwell (not standard).
    """
    existing_surface_guid: str
    proposed_surface_guid: str
    cut_volume_m3: float                           # undisturbed (bank) cubic meters
    fill_volume_m3: float                          # in-place (compacted) cubic meters
    net_volume_m3: float                           # cut - fill; positive = net excavation
    cut_cubic_yards: float
    fill_cubic_yards: float
    net_cubic_yards: float
    shrink_factor: float = 1.0                     # fill-side shrinkage (ratio compacted/bank)
    swell_factor: float = 1.0                      # cut-side swell (ratio loose/bank)
    compacted_fill_m3: float = 0.0                 # fill_volume_m3 (already compacted by definition)
    loose_cut_m3: float = 0.0                      # cut_volume_m3 * swell_factor
    per_triangle_deltas: np.ndarray | None = None  # optional, for cut/fill color map
    color_map_mesh_id: int | None = None           # Blender mesh id for overlay

## 6. Workflow Reference

### 6.1 Dataflow diagram

┌──────────────┐     ┌──────────────┐     ┌───────────────┐
│ Existing     │     │ Feature      │     │ Grading       │
│ Ground       │     │ Lines        │     │ Criteria      │
│ (CivilSurf)  │     │ (IfcAlign.)  │     │ (Pset tmpl)   │
└──────┬───────┘     └──────┬───────┘     └───────┬───────┘
       │                    │                     │
       │                    ▼                     │
       │            ┌──────────────┐              │
       └───────────▶│ Grading      │◀─────────────┘
                    │ Object       │
                    │ (FL+Crit)    │
                    │              │
                    │ → IfcEWFill  │
                    │   [SLOPEFILL]│
                    └──────┬───────┘
                           │ N objects per group
                           ▼
                    ┌──────────────┐
                    │ Grading      │
                    │ Group        │◀── Interior fill strategy
                    │              │        │
                    │ → IfcGroup   │        ▼
                    │   ObjectType │    IfcEWFill
                    │   ="Grading  │    [SUBGRADE]
                    │   Group"     │    (interior)
                    └──────┬───────┘
                           │
                           ▼
                    ┌──────────────┐     ┌──────────────┐
                    │ Composite    │     │ Existing     │
                    │ Proposed     │     │ Surface      │
                    │ Surface      │     │              │
                    │ (IfcEWFill   │     │ (IfcGeotech  │
                    │  composite)  │     │  Stratum)    │
                    └──────┬───────┘     └──────┬───────┘
                           │                    │
                           └─────────┬──────────┘
                                     ▼
                            ┌──────────────────┐
                            │ Earthwork        │
                            │ Volumes (TIN-TIN)│
                            └────────┬─────────┘
                                     ▼
                            ┌──────────────────┐
                            │ Qto sets written:│
                            │ - Qto_EWCut...   │
                            │ - Qto_EWFill...  │
                            │ Cut/Fill solids  │
                            │ as IfcPolygonal  │
                            │ FaceSet Closed   │
                            └──────────────────┘

### 6.2 Slope projection algorithm (criteria-to-surface)

This is the central grading computation. It runs once when a GradingObject is added to a group, and re-runs whenever the feature line is edited.

For each segment between consecutive vertices of the feature line:
    For each sample point along the segment (spaced by configured interval):
        1. Get footprint point P₀ = (x, y, z) at the sample position on feature line.
        2. Query target surface Z at (x, y) → z_target.
        3. If z_target > z₀: this is a cut (feature line below ground).
           Use cut_slope; project outward horizontally-outward + upward.
        4. If z_target < z₀: this is a fill (feature line above ground).
           Use fill_slope; project outward horizontally-outward + downward.
        5. March outward at the chosen slope until one of:
             a. Intersection with target surface (daylight point P_tie).
             b. max_distance reached — insert retaining wall if configured,
                otherwise terminate slope at that distance.
        6. Record P_tie as a point on the daylight line.
    Triangulate the ribbon between the feature line segment and its corresponding
    daylight segment — two triangles per sample step, respecting the outward
    normal convention.

Outward direction: for closed feature lines (pad perimeters), outward is “away from the interior” — determined by polygon orientation (counterclockwise = outward is to the right of the direction of traversal). For open feature lines, the user specifies which side the projection goes on (matches Civil 3D’s “Apply to Side” prompt).

### 6.3 Grading group composition

When a group has multiple grading objects, its composite proposed surface is built from:

- All feature line vertices (at their authored elevations).

- All daylight line vertices (at their computed elevations).

- All projection ribbon triangles (one set per grading object → IfcEarthworksFill[SLOPEFILL]).

- Interior fill triangles, per the interior_fill strategy → IfcEarthworksFill[SUBGRADE]:

  - none — no interior, surface has a hole. No interior fill entity authored.

  - flat — interior is a flat surface at the average feature line elevation.

  - interpolate_from_boundary — Delaunay triangulation using feature line vertices only.

  - from_surface — use another named surface for the interior (useful for pit-bottom or pre-designed pad-bottom cases).

The composite is authored as an IfcEarthworksFill[SUBGRADE] entity that aggregates the slope fills and interior fill via IfcRelAggregates. It carries both the proposed TIN (as SurfaceModel representation) and the composite fill solid (as Brep representation).

### 6.4 TIN-to-TIN prismoidal volumes

Civil 3D’s most accurate volume method (its Volumes Dashboard → Composite Volumes). The implementation:

Input: surface_existing, surface_proposed (both CivilSurface)
Output: cut_volume, fill_volume

1. Compute domain = outer_boundary(existing) ∩ outer_boundary(proposed)
   minus holes/voids from either.
2. For each pair of triangles (t_e ∈ existing, t_p ∈ proposed) whose XY projections
   overlap within the domain:
     a. intersection_polygon = t_e.xy ∩ t_p.xy ∩ domain
     b. Triangulate intersection_polygon → sub-triangles.
     c. For each sub-triangle s:
          z_e_avg = mean of existing surface Z at s's three vertices (interpolated)
          z_p_avg = mean of proposed surface Z at s's three vertices (interpolated)
          delta_z = z_e_avg - z_p_avg
          volume = area(s) * delta_z          # signed
          if delta_z > 0: cut_volume += volume
          else:           fill_volume += (-volume)
3. Return cut_volume, fill_volume (both positive magnitudes).

Grid-based alternatives are faster but lose fidelity on narrow features — MVP uses TIN prismoidal only.

### 6.5 Cut/fill solid construction

After volume calculation, Saikei constructs closed IfcPolygonalFaceSet solids for the cut and fill volumes. Per region (contiguous cut or contiguous fill within the domain):

For a fill region:
  1. Top face: the portion of the proposed TIN over the region
     (triangles from proposed_surface whose XY lies in the region).
  2. Bottom face: the portion of the existing TIN under the region
     (triangles from existing_surface, reversed winding).
  3. Side faces: triangulated strips between top and bottom boundaries
     (the projection of the region boundary onto both surfaces).
  4. Assemble all triangles into a single IfcPolygonalFaceSet with Closed=TRUE.

Cut regions mirror this (existing is "top", proposed is "bottom").

The result is a watertight closed BRep solid that round-trips through any IFC viewer supporting IfcPolygonalFaceSet.

## 7. Core and Tool API

### 7.1 Core — orchestration only

Core does not invent IFC API paths. It delegates all IFC work to tool methods.

# core/surface.py

def create_surface_from_points(tool, name: str, points: np.ndarray, kind: str) -> str:
    """Create a CivilSurface from a point cloud. Returns surface guid."""
    surface = tool.Surface.build_tin_from_points(name, points, kind)
    tool.Surface.author_ifc_host(surface)              # creates IfcGeotechnicalStratum or IfcEarthworksFill
    tool.Surface.author_ifc_tin_representation(surface)
    tool.Surface.author_ifc_bounding_box(surface)
    tool.Surface.apply_standard_psets(surface)
    tool.Surface.apply_classification(surface)
    tool.Surface.link_to_blender(surface)
    return surface.guid


def add_breakline_to_surface(tool, surface_guid: str, breakline: Breakline) -> None:
    """Add a breakline to a surface, persist as IfcAnnotation, and retriangulate."""
    surface = tool.Surface.get(surface_guid)
    tool.Surface.add_breakline(surface, breakline)
    tool.Surface.author_ifc_annotation(breakline)      # IfcAnnotation + IfcPolyline
    tool.Surface.retriangulate(surface)
    tool.Surface.update_ifc_tin_representation(surface)
    tool.Surface.update_blender(surface)


def set_surface_boundary(tool, surface_guid: str, polygon, kind: str) -> None:
    """Set outer boundary, add a hole, or add a void."""
    surface = tool.Surface.get(surface_guid)
    tool.Surface.set_boundary(surface, polygon, kind)
    tool.Surface.retriangulate(surface)
    tool.Surface.update_ifc_tin_representation(surface)
    tool.Surface.update_blender(surface)


# core/grading.py

def create_grading_group(tool, name: str, target_surface_guid: str,
                         interior_fill: str = "interpolate_from_boundary") -> str:
    """Create an empty grading group targeting an existing surface."""
    target = tool.Surface.get(target_surface_guid)
    group = tool.Grading.create_group(name, target, interior_fill)
    tool.Grading.author_ifc_group(group)               # IfcGroup ObjectType="GradingGroup"
    tool.Grading.author_ifc_composite_fill(group)      # IfcEarthworksFill [SUBGRADE] (composite)
    tool.Grading.apply_source_pset(group)              # Pset_SaikeiGradingSource
    return group.guid


def create_grading_criteria(tool, name: str, target_kind: str, cut_slope: float,
                              fill_slope: float, max_distance: float | None = None) -> str:
    """Author a reusable grading criteria as an IfcPropertySetTemplate."""
    criteria = tool.Grading.build_criteria(name, target_kind, cut_slope, fill_slope, max_distance)
    tool.Grading.author_ifc_criteria_template(criteria)
    return criteria.guid


def add_grading_object(tool, group_guid: str, feature_line_guid: str,
                       criteria_guid: str) -> str:
    """Apply a criteria to a feature line within a group."""
    group = tool.Grading.get_group(group_guid)
    feature_line = tool.Grading.get_feature_line(feature_line_guid)
    criteria = tool.Grading.get_criteria(criteria_guid)
    target = tool.Surface.get(group.target_surface_guid)
    
    grading_object = tool.Grading.compute_grading_object(
        feature_line, criteria, target
    )
    tool.Grading.author_ifc_slope_fill(grading_object) # IfcEarthworksFill [SLOPEFILL]
    tool.Grading.add_to_group(group, grading_object)
    tool.Grading.rebuild_group_surface(group)
    tool.Grading.update_blender(group)
    return grading_object.guid


def edit_feature_line_elevations(tool, feature_line_guid: str,
                                   edits: list[tuple[int, float]]) -> None:
    """Edit feature line vertex elevations; cascade rebuild to all parent groups."""
    feature_line = tool.Grading.get_feature_line(feature_line_guid)
    tool.Grading.apply_elevation_edits(feature_line, edits)
    tool.Grading.update_ifc_alignment_representation(feature_line)
    
    # Dynamic rebuild — find all grading objects using this feature line
    affected_groups = tool.Grading.find_groups_using_feature_line(feature_line_guid)
    for group in affected_groups:
        tool.Grading.rebuild_group_surface(group)
        tool.Grading.update_blender(group)


# core/earthwork.py

def compute_earthwork_volumes(tool, existing_guid: str, proposed_guid: str,
                                shrink_factor: float = 1.0,
                                swell_factor: float = 1.0) -> VolumeResult:
    """Compute cut and fill volumes between two surfaces."""
    existing = tool.Surface.get(existing_guid)
    proposed = tool.Surface.get(proposed_guid)
    result = tool.Earthwork.volume_tin_to_tin(
        existing, proposed, shrink_factor, swell_factor
    )
    tool.Earthwork.author_cut_solid(result)            # IfcEarthworksCut + IfcPolygonalFaceSet
    tool.Earthwork.author_fill_solid(result)           # IfcEarthworksFill + IfcPolygonalFaceSet
    tool.Earthwork.write_standard_qtos(result)         # Qto_EarthworksCutBaseQuantities, Qto_EarthworksFillBaseQuantities
    tool.Earthwork.write_shrink_swell_pset(result)     # Pset_SaikeiGradingShrinkSwell
    tool.Earthwork.apply_classification(result)        # OmniClass Table 22
    return result


def generate_cutfill_map(tool, existing_guid: str, proposed_guid: str) -> int:
    """Generate a false-color overlay mesh showing cut (red) and fill (blue)."""
    existing = tool.Surface.get(existing_guid)
    proposed = tool.Surface.get(proposed_guid)
    mesh_id = tool.Earthwork.build_cutfill_color_mesh(existing, proposed)
    return mesh_id

### 7.2 Tool — implementations

Tool methods author IFC entities directly via ifc_file.create_entity(). Examples:

# tool/surface.py

class Surface:
    @staticmethod
    def build_tin_from_points(name: str, points: np.ndarray, kind: str) -> CivilSurface: ...

    @staticmethod
    def retriangulate(surface: CivilSurface) -> None:
        """Rebuild the TIN using constrained Delaunay. Honors breaklines, holes, voids.
        Updates surface.triangles and surface.triangle_flags in place."""

    @staticmethod
    def z_at(surface: CivilSurface, x: float, y: float) -> float | None:
        """Interpolated Z at (x, y), or None if outside. Point-in-triangle +
        barycentric interpolation."""

    @staticmethod
    def author_ifc_host(surface: CivilSurface) -> None:
        """Create the appropriate IFC host entity based on surface.kind.
        
        existing       → IfcGeotechnicalStratum under IfcGeomodel via IfcRelAggregates
        proposed_group → IfcEarthworksFill [SUBGRADE] within a grading group
        proposed_site  → IfcEarthworksFill [SUBGRADE] under IfcGeomodel via IfcRelAggregates
        
        All use IfcLocalPlacement with identity IfcAxis2Placement3D."""
        ifc_file = tool.Ifc.get()
        # ... create host entity with identity placement ...
        # ... attach to IfcGeomodel via IfcRelAggregates ...
        # ... stash step id in surface.ifc_host_entity_id ...

    @staticmethod
    def author_ifc_tin_representation(surface: CivilSurface) -> None:
        """Author the IfcTriangulatedIrregularNetwork and attach as a SurfaceModel
        representation on the host entity."""
        ifc_file = tool.Ifc.get()
        coord_list = ifc_file.create_entity(
            "IfcCartesianPointList3D",
            CoordList=[tuple(p) for p in surface.points],
        )
        tin = ifc_file.create_entity(
            "IfcTriangulatedIrregularNetwork",
            Coordinates=coord_list,
            CoordIndex=[tuple(t + 1) for t in surface.triangles],  # 1-based
            Flags=surface.triangle_flags.tolist(),
            Closed=False,
        )
        # Wrap in IfcShapeRepresentation with RepresentationIdentifier='SurfaceModel'
        # Attach to host entity's IfcProductDefinitionShape.Representations
        surface.ifc_tin_representation_id = tin.id()

    @staticmethod
    def author_ifc_bounding_box(surface: CivilSurface) -> None:
        """Author an IfcBoundingBox representation for LOD/culling."""
        ...

    @staticmethod
    def author_ifc_annotation(breakline: Breakline) -> None:
        """Persist a breakline as IfcAnnotation with IfcPolyline representation.
        Separate from the derived TIN — enables retriangulation on re-open."""
        ...

    @staticmethod
    def apply_standard_psets(surface: CivilSurface) -> None:
        """Attach Pset_GeotechnicalStratumTypeCommon (for existing) or
        Pset_EarthworksFillCommon (for proposed) via IfcRelDefinesByProperties.
        Also apply Pset_SaikeiGradingSurface for Saikei-specific metadata."""
        ...

    @staticmethod
    def apply_classification(surface: CivilSurface) -> None:
        """Attach OmniClass Table 22 classification via IfcRelAssociatesClassification."""
        ...


# tool/grading.py

class Grading:
    @staticmethod
    def compute_grading_object(feature_line: FeatureLine,
                                criteria: GradingCriteria,
                                target_surface: CivilSurface | None) -> GradingObject:
        """Dispatch by criteria.target_kind to the four projection implementations."""

    @staticmethod
    def author_ifc_group(group: GradingGroup) -> None:
        """Create IfcGroup with ObjectType='GradingGroup'.
        Attach to IfcSite via IfcRelContainedInSpatialStructure."""

    @staticmethod
    def author_ifc_composite_fill(group: GradingGroup) -> None:
        """Create the composite IfcEarthworksFill [SUBGRADE] that will aggregate
        per-object slope fills and the interior fill."""

    @staticmethod
    def author_ifc_slope_fill(grading_object: GradingObject) -> None:
        """Create IfcEarthworksFill [SLOPEFILL] for the slope projection region.
        Attach as member of the group via IfcRelAggregates to the composite fill."""

    @staticmethod
    def author_ifc_criteria_template(criteria: GradingCriteria) -> None:
        """Create IfcPropertySetTemplate at project scope for reusable criteria.
        Instances of this template (IfcPropertySet) are attached to each group that
        uses the criteria."""

    @staticmethod
    def rebuild_group_surface(group: GradingGroup) -> CivilSurface:
        """Compose all gradings + interior fill into a single proposed CivilSurface.
        Author the interior fill as IfcEarthworksFill [SUBGRADE] if interior_fill != 'none'."""


# tool/earthwork.py

class Earthwork:
    @staticmethod
    def volume_tin_to_tin(existing: CivilSurface, proposed: CivilSurface,
                           shrink_factor: float = 1.0,
                           swell_factor: float = 1.0) -> VolumeResult:
        """True TIN-to-TIN prismoidal volume. See §6.4."""

    @staticmethod
    def author_cut_solid(result: VolumeResult) -> None:
        """Author IfcEarthworksCut with IfcPolygonalFaceSet Closed=TRUE.
        Establish IfcRelVoidsElement relationship to the host IfcGeotechnicalStratum."""

    @staticmethod
    def author_fill_solid(result: VolumeResult) -> None:
        """Author IfcEarthworksFill with IfcPolygonalFaceSet Closed=TRUE in a Brep
        ShapeRepresentation alongside the proposed TIN SurfaceModel representation."""

    @staticmethod
    def write_standard_qtos(result: VolumeResult) -> None:
        """Write volumes to Qto_EarthworksCutBaseQuantities and
        Qto_EarthworksFillBaseQuantities. This is where QTO tools find them."""

    @staticmethod
    def build_cutfill_color_mesh(existing: CivilSurface, proposed: CivilSurface) -> int:
        """Generate a Blender mesh overlay with per-vertex colors driven by Z delta."""

## 8. UI Layer

### 8.1 Panels

- CIVIL_PT_surface_panel — surface list, create/edit, visualization toggles (triangles, contours, slope vectors, elevation banding).

- CIVIL_PT_grading_panel — grading groups list; per-group shows members, target surface, interior fill strategy, cut/fill totals.

- CIVIL_PT_earthwork_panel — earthwork volume calculation, cut/fill color map controls, shrink/swell factor inputs.

### 8.2 Operators (MVP set)

Surface: - CIVIL_OT_surface_create_from_points — import XYZ, CSV, or IFC TIN; build surface. - CIVIL_OT_surface_add_breakline — pick a 3D polyline in viewport, convert to breakline. - CIVIL_OT_surface_set_boundary — pick a 2D polygon as outer boundary, hole, or void. - CIVIL_OT_surface_retriangulate — force rebuild (rarely needed manually). - CIVIL_OT_surface_export_ifc — write IfcTriangulatedIrregularNetwork + bbox + breakline annotations to IFC.

Grading: - CIVIL_OT_feature_line_create — draw a new feature line or convert mesh edge loop. Authors IfcAlignment. - CIVIL_OT_feature_line_drape — assign vertex elevations from a surface. - CIVIL_OT_feature_line_edit_elevations — vertex-by-vertex elevation edit dialog. - CIVIL_OT_grading_create_group — create new group; prompts for name, target surface, interior fill. - CIVIL_OT_grading_create_criteria — author new criteria as IfcPropertySetTemplate. - CIVIL_OT_grading_add_object — apply criteria to feature line within a group. - CIVIL_OT_grading_rebuild_group — force-rebuild group surface (auto-rebuild handles most cases).

Earthwork: - CIVIL_OT_earthwork_compute_volumes — pair two surfaces, compute and display cut/fill. Writes standard Qtos. - CIVIL_OT_earthwork_cutfill_map — generate cut/fill color overlay mesh. - CIVIL_OT_earthwork_export_ifc — write IfcEarthworksCut / IfcEarthworksFill solids (IfcPolygonalFaceSet Closed) with classification and Qtos.

## 9. MVP Scope — Sprint 1

Keeping PRs ≤4,000 lines (Dion’s constraint), Sprint 1 is “Terrain Modeler” only. Grading and earthwork follow in Sprints 2 and 3.

In scope for Sprint 1: - CivilSurface data model and tool.Surface implementation - TIN construction from point cloud (unconstrained Delaunay via SciPy) - TIN construction with breaklines (CDT via Shapely 2.1+) - Outer boundary + holes + voids - IfcTriangulatedIrregularNetwork authoring + IfcBoundingBox + IfcAnnotation breaklines - Host entity authoring (IfcGeotechnicalStratum for existing surfaces) - IfcGeomodel containment via IfcRelContainedInSpatialStructure; strata aggregated via IfcRelAggregates - IfcMapConversion + IfcProjectedCRS.VerticalDatum wiring - Standard psets applied: Pset_GeotechnicalStratumTypeCommon, Pset_SoilMechanicalLayer - Custom psets: Pset_SaikeiGradingSurface - OmniClass Table 22 classification - Basic Blender visualization (mesh with vertex colors for elevation banding) - Surface panel + create-from-points + add-breakline + set-boundary operators - Round-trip tests (write → read → compare) through ifcopenshell - Validation test using bSI reference validator (see §11)

Deferred to Sprint 2: - Feature lines data model + UI + IfcAlignment authoring with polyline representation - Grading criteria as IfcPropertySetTemplate - Grading objects (the four projection methods) + IfcEarthworksFill[SLOPEFILL] authoring - Grading groups + IfcGroup authoring + composite surfaces + interior fill as IfcEarthworksFill[SUBGRADE] - Pset_SaikeiGradingAlignment for linking grading groups to corridor alignments - Cascade-on-edit behavior (Civil 3D’s dynamic grading)

Deferred to Sprint 3: - Earthwork volumes (TIN-to-TIN prismoidal) - Cut/fill color map overlay - IfcEarthworksCut authoring with IfcPolygonalFaceSet Closed solid and IfcRelVoidsElement - IfcEarthworksFill closed solid (Brep) alongside proposed TIN - Standard Qto authoring: Qto_EarthworksCutBaseQuantities, Qto_EarthworksFillBaseQuantities - Shrink/swell factors in Pset_SaikeiGradingShrinkSwell

Deferred beyond MVP: - LandXML import/export - Retaining walls at daylight cap (requires IfcWall integration — non-TIN geometry) - Mass haul diagrams - Full IfcLinearPlacement integration with corridor alignments (Pset-based in MVP)

## 10. Known Limitations (acknowledged, not bugs)

Overhanging and vertical surfaces. IfcTriangulatedIrregularNetwork requires one unique Z per (X, Y). Retaining walls, steep cut faces, and certain pond cross-sections have vertical or overhanging faces that cannot be represented. For MVP these are out of scope; when first encountered in practice, they will be handled via IfcWall entities (retaining walls) placed at the daylight line, separate from the TIN.

Breakline edge identity in Flags list. IFC’s per-triangle flag cannot preserve per-edge breakline identity when a triangle straddles a breakline. Round-trip preserves triangle membership (is this triangle adjacent to breakline N?) but not which specific edge is on the breakline. Full breakline source geometry is persisted separately as IfcAnnotation polylines so this isn’t a data-loss issue — just a query-efficiency one.

Material flow between cut and fill. IFC 4.3 does not model excavated material as a resource, so the relationship between a cut at one station and a fill at another (where the dirt moved) cannot be expressed in the schema. Mass haul optimization is a future module that will use Saikei-native property extensions until IFC provides the concept.

Third-party viewer support for IFC 4.3 civil entities. IfcTriangulatedIrregularNetwork, IfcEarthworksCut, IfcEarthworksFill, IfcGeomodel have partial-to-absent support in building-focused viewers (BIMvision, Solibri, Navisworks as of mid-2025). Interoperability for Saikei output is primarily with IFC 4.3 civil tools — OpenRoads, DESITE, bSI reference validator, Revit with latest Roads/Rail extensions.

## 11. Agent Work Breakdown

### Sprint 1 — Surfaces (target: ~2,500 LoC)

| Agent | Task | Est. LoC |
| --- | --- | --- |
| saikei-architect | Finalize CivilSurface + Breakline data models, triangulator abstraction, IFC host selection logic | 400 |
| saikei-ifc | IfcGeotechnicalStratum + IfcGeomodel + IfcTriangulatedIrregularNetwork + IfcBoundingBox + IfcAnnotation authoring, standard psets, OmniClass classification, round-trip tests | 800 |
| tool-dev | tool.Surface impl — build_tin_from_points, retriangulate with CDT, z_at, boundary/hole/void handling | 700 |
| blender-ui | Surface panel, operators for create-from-points, add-breakline, set-boundary | 400 |
| saikei-tester | Test TIN build, CDT with breaklines, IFC round-trip, bSI reference validator run | 200 |
| docs | Surface module README, CLAUDE.md update | 100 |

### Sprint 2 — Grading (target: ~3,000 LoC)

| Agent | Task | Est. LoC |
| --- | --- | --- |
| saikei-architect | Finalize FeatureLine, GradingCriteria, GradingObject, GradingGroup data models; alignment-linkage pset design | 400 |
| saikei-ifc | IfcAlignment authoring for feature lines, IfcPropertySetTemplate for criteria, IfcGroup + IfcEarthworksFill[SLOPEFILL/SUBGRADE] authoring, Pset_SaikeiGradingAlignment | 600 |
| tool-dev | tool.Grading impl — all four projection methods, group rebuild, cascade edit | 1000 |
| blender-ui | Feature line authoring, grading panel, criteria dialog, add-object operator | 700 |
| saikei-tester | Test pad grading, pond-like grading, group composition, cascade edit, round-trip | 200 |
| docs | Grading module README + tutorial (flat pad to existing ground) | 100 |

### Sprint 3 — Earthwork (target: ~2,000 LoC)

| Agent | Task | Est. LoC |
| --- | --- | --- |
| saikei-architect | VolumeResult schema, earthwork report structure, cut/fill solid construction algorithm | 200 |
| saikei-ifc | IfcEarthworksCut + IfcEarthworksFill solid authoring (IfcPolygonalFaceSet Closed), IfcRelVoidsElement, standard Qto authoring, Pset_SaikeiGradingShrinkSwell | 500 |
| tool-dev | tool.Earthwork impl — TIN-to-TIN prismoidal, cut/fill region extraction, closed-solid construction, color map | 700 |
| blender-ui | Earthwork panel, compute-volumes operator, cut/fill map operator | 400 |
| saikei-tester | Test volumes against hand-calc values; IFC round-trip through DESITE or bSI reference validator | 150 |
| docs | Earthwork README + workflow example (full pad → cut/fill report → IFC export) | 50 |

## 12. Open Questions — Remaining

Most previous open questions have been resolved by this spec revision. Remaining items for Dion / Rick:

- IfcAlignment as feature line representation — upstream acceptance. Using IfcAlignment for 3D grading feature lines is schema-valid (the spec explicitly lists IfcPolyline as a valid alignment representation), but it’s a semantic stretch — alignments are typically “transportation routes.” Confirm with Dion that this reuse is acceptable before Sprint 2 commits to it. Alternative: IfcAnnotation with 3D polyline representation, less sophisticated but unambiguous.

- Module prefix collisions. CIVIL_OT_surface_* — check against any existing Bonsai site module that might overlap semantically.

- Georeferencing precision on large surfaces. Alignments are linear and stay close to the alignment envelope; surfaces span the whole site. Verify tool.Georeference ENH↔XYZ behavior is accurate out to a several-km site extent without precision drift. A small test harness in Sprint 1 answers this.

- Vertical datum — ecosystem behavior. We’re prescribing IfcProjectedCRS.VerticalDatum + IfcMapConversion.OrthogonalHeight for vertical round-trip. Worth a survey of what Bonsai alignment currently does and whether fixing it for surfaces warrants a coordinated fix for alignments too.

- Retaining wall placeholder. Spec defers retaining walls beyond MVP. Confirm this aligns with Dion’s upstream priorities — if a highway project comes through before we implement walls, the grading module will need a fallback.

## 13. References

- buildingSMART IFC 4.3.2 — IfcTriangulatedIrregularNetwork

- buildingSMART IFC 4.3.2 — IfcEarthworksCut

- buildingSMART IFC 4.3.2 — IfcEarthworksFill

- buildingSMART IFC 4.3.2 — IfcGeomodel

- buildingSMART IFC 4.3.2 — IfcGeotechnicalStratum

- buildingSMART IFC 4.3.2 — IfcPolygonalFaceSet

- buildingSMART IFC 4.3.2 — IfcGroup

- buildingSMART IFC 4.3.2 — [Qto_EarthworksFillBaseQuantities, Qto_EarthworksCutBaseQuantities]

- IRROADWP3 Conceptual Model Report Annex I — Earthworks instance diagrams (project knowledge)

- BIM Corner — IFC 4.3 in road projects – case study (confirms IfcRelContainedInSpatialStructure for IfcGeomodel under IfcSite)

- OmniClass Construction Classification System — Table 22 Work Results

- Autodesk Civil 3D Help — Creating Gradings, Grading Objects reference

- Shapely 2.1 — constrained_delaunay_triangles

- SciPy — scipy.spatial.Delaunay

- Prior Saikei research — Corridor_Generation_Deep_Research.md, IFC_Roadway_Templates_Assemblies_Reference.md, saikei-project-summary.md

- Companion research — Saikei_Grading_Earthwork_Research.md (commercial tool comparative analysis)

End of implementation spec. Ready for Sprint 1 agent work.